Reading NISAR Products with GDAL

A Practical How-To Guide for the NISAR HDF5 GDAL Driver

All product types: RSLC · RIFG · RUNW · GSLC · GCOV · GUNW · GOFF · SME2 · radar-grid metadata cubes

Prepared for NISAR science data users

21 September 2026

Reflects gdal-driver-nisar version 0.6.6 (built against GDAL 3.12). Behaviour statements have been checked against the driver source, the repository README and the nisar-gdal skill document; anything that could not be re-verified is marked observed or historical.

# Contents

1. About this guide

2. Installation and verification

3. Data access paths and credentials

4. The addressing model: containers, subdatasets, open options

5. Exploring a granule

6. Open option reference

7. Per-product recipes

7.1 RSLC, RIFG, RUNW · 7.2 GSLC · 7.3 GCOV · 7.4 GUNW and GOFF · 7.5 SME2 · 7.6 Radar-grid metadata cubes · 7.7 Static layers

8. Reprojection and warping to latitude-longitude

9. Masking

10. Derived subdatasets and virtual overviews

11. Performance

12. Python (osgeo.gdal)

13. Troubleshooting

14. Version history and behaviour changes

15. Verification checklist

16. Quick reference

# 1. About this guide

The NISAR GDAL driver (conda package gdal-driver-nisar, distributed through the nisar-forge channel, source in the nisar-hdf5-gdalplugin repository) lets you read NISAR HDF5 granules directly with the standard GDAL command-line tools, and with anything built on GDAL: QGIS, rasterio, TiTiler and the osgeo.gdal Python bindings. You do not need to download whole granules, convert them to GeoTIFF first, or write HDF5 code. A single gdalinfo, gdal_translate or gdalwarp invocation can reach into a granule sitting in an S3 bucket, pull only the byte ranges it needs, and hand you a georeferenced raster.

The driver is read-only, registers in GDAL under the short name `NISAR`, and installs one shared library (gdal_NISAR.so on Linux, gdal_NISAR.dylib on macOS) into GDAL's plugin directory. It is not a Python package and there is nothing to import; once installed, every GDAL tool in that environment gains NISAR support.

This guide is a task-oriented reference. It assumes you are comfortable with SAR and InSAR concepts and with GDAL itself; it does not explain what a covariance matrix or an unwrapped interferogram is. What it does explain is how the driver maps NISAR HDF5 structure onto GDAL's dataset model (container → subdatasets, 3-D cube → bands, mask dataset → mask band, GCPs versus GeoTransform), which open options apply to which product, and where the sharp edges are.

## 1.1 How to read the command examples

Every command in this guide is a complete, runnable line. Where a command is too long for the page it is broken with a trailing backslash, which is a shell line continuation, so you can paste the whole block into bash or zsh as it stands. Where a value is a placeholder it is written in angle brackets.

Two conventions matter throughout. First, the driver is addressed through a NISAR: connection-string prefix, described in section 4. Second, most behaviour is controlled by GDAL open options passed as -oo KEY=VALUE, and these are the main thing to learn.

NISAR granule names are long enough to make a command unreadable, so the examples refer to granules through shell variables. Set them once at the start of a session and every command in this guide will run as written. The paths below are real paths from an operational bucket at the time of writing, but they will age out; replace them with your own granules, keeping the variable names.

In the operational buckets each granule sits in a directory named after the granule itself, so the paths can be built from a bucket prefix and a granule ID:

B=/vsis3/nisar-ops-rs-fwd/products

G=NISAR_L2_PR_GCOV_006_068_D_093_4005_DHDH_A_20251009T222629_20251009T222704_P00410_N_F_J_001

export GCOV=$B/L2_L_GCOV/2025/10/09/$G/$G.h5

G=NISAR_L1_PR_RSLC_006_068_D_093_4005_DHDH_A_20251009T222629_20251009T222704_P00410_N_F_J_001

export RSLC=$B/L1_L_RSLC/2025/10/09/$G/$G.h5

G=NISAR_L2_PR_GSLC_006_019_A_018_4005_DHDH_A_20251006T120932_20251006T121008_P00410_N_F_J_001

export GSLC=$B/L2_L_GSLC/2025/10/06/$G/$G.h5

G=NISAR_L3_PR_SME2_006_019_A_018_4005_DHDH_A_20251006T120932_20251006T121008_P00410_N_F_J_001

export SME2=$B/L3_L_SME2/2025/10/06/$G/$G.h5

# GUNW granule IDs carry two acquisition date-time pairs, so they are longer still

G=NISAR_L2_PR_GUNW_006_019_A_018_007_4000_SH_20251006T120932_20251006T121008_\

20251018T120933_20251018T121008_P05000_N_F_J_001

export GUNW=$B/L2_L_GUNW/2025/10/06/$G/$G.h5

# global public DEM, used for metadata cube interpolation in section 7.6

export DEM=/vsis3/sds-n-cumulus-prod-nisar-products/DEM/v1.2/EPSG4326/EPSG4326.vrt

The variables hold bare paths, without the NISAR: prefix, so the same variable works both on its own and with an HDF5 dataset path appended. Note that the GUNW assignment above is split across two lines with a backslash purely to fit the page; the granule ID is a single unbroken string.

## 1.2 What is verified and what is not

This edition was redrafted against the driver source for version 0.6.6, the repository README.md and the nisar-gdal skill document. Three kinds of statement appear, and the guide tries to keep them apart:

Verified in source. How the connection string is parsed, which open options exist and what they do, how georeferencing, masks, statistics and derived subdatasets are produced. These are stated plainly.

*Observed.* Behaviour seen on a specific granule with a specific driver build (timings, memory figures, a warning printed on one product). Marked as such; not re-run for this edition.

*Historical.* Behaviour of earlier releases, kept only where it explains why an old command or an inherited pipeline behaves differently. See section 14.

HDF5 layer names inside a product (for example the exact spelling of a GUNW layer) come from the NISAR product specifications, and vary with product type, instrument and specification version.

Rule of thumb. Never trust a layer path from any document, including this one. Run gdalinfo on the granule first and copy the SUBDATASET_n_NAME string verbatim. Section 5 shows how.

# 2. Installation and verification

## 2.1 Installing the driver

The driver ships as a conda package that places the plugin shared library into $CONDA_PREFIX/lib/gdalplugins, the directory GDAL scans for plugins. The package is pinned to the GDAL minor version it was built against (3.12 for 0.6.6) and pulls in compatible gdal, libgdal-core and hdf5 packages. Supported platforms are Linux x86_64, Linux aarch64 and macOS arm64; Windows is not supported.

To create a dedicated environment, which is the recommended approach:

mamba create -n nisar-env -c nisar-forge -c conda-forge gdal-driver-nisar

conda activate nisar-env

To add or update the driver inside an environment you already have:

mamba install -c nisar-forge -c conda-forge gdal-driver-nisar

If conda refuses to solve, the GDAL already in that environment is almost certainly a different minor version from the one the plugin was built against. Do not fight the solver; install into a fresh environment instead.

Pin an exact version in any processing pipeline whose outputs you intend to compare over time:

mamba install -c nisar-forge -c conda-forge gdal-driver-nisar=0.6.6

Building from source (natively on macOS, or for Linux through Docker) is documented in the repository's BUILDING.md. The plugin is a CMake project in conda-build/nisar-gdal-recipe/; if you install it somewhere GDAL does not scan, point GDAL_DRIVER_PATH at that directory.

## 2.2 The stale-version trap

This deserves its own subsection because it has caught users repeatedly, and because it fails silently rather than loudly. An install or update can report success while conda list continues to show the old version and the old plugin binary stays in place (observed). The symptom is not an error; it is wrong output. In one documented case a user spent time debugging an 80 m pixel size that looked like frequency B data, when in fact the driver was simply an old build with a broken GeoTransform.

Always confirm the version from the loaded binary after installing (section 2.3), and if it has not moved, remove the package outright and install it again rather than reaching for --force-reinstall a second time:

gdalinfo --format NISAR | grep DRIVER_VERSION

# if the version is not what you expect:

conda remove gdal-driver-nisar

mamba install -c nisar-forge -c conda-forge gdal-driver-nisar

gdalinfo --format NISAR | grep DRIVER_VERSION

## 2.3 Verifying the driver loaded

Three checks, in increasing specificity. The first confirms GDAL can see the plugin at all:

gdalinfo --formats | grep NISAR

Expected output (the capability flags may differ slightly between GDAL builds):

NISAR -raster- (rovs): NISAR HDF5 (*.h5)

The second confirms the plugin does not have a broken dynamic link. This matters because a plugin that fails to load prints its error on every GDAL invocation, including ones that have nothing to do with NISAR:

gdalinfo --version

If that command emits a GLIBC error rather than a version string, the plugin binary is newer than your system C library. Older hosts, including some JupyterHub base images, produce an error naming the exact missing symbol version. The fix is a driver rebuild for your platform, not a configuration change on your side.

# symptom of a GLIBC mismatch:

ERROR 1: /lib64/libc.so.6: version `GLIBC_2.38' not found

(required by .../lib/gdalplugins/gdal_NISAR.so)

The third check asks the driver to report its own version and build date, and also prints the list of open options it registers. This is the authoritative answer to the question of which driver you are running, and it is more trustworthy than conda list because it comes from the binary that is actually loaded:

gdalinfo --format NISAR

Look for a DRIVER_VERSION entry of the form v0.6.6 (Build Date: ...).

# 3. Data access paths and credentials

## 3.1 The four ways to point at a granule

The driver reads local files and remote objects through the same interface. All remote I/O goes through GDAL's Virtual File System (VSI): the driver installs a custom HDF5 Virtual File Layer so that every HDF5 read becomes a VSIFReadL() call, and GDAL performs the HTTP range requests, caching, retries and authentication. There is no separate HDF5 ROS3 code path any more, so the same AWS_* and GDAL_HTTP_* settings you use for any other GDAL driver apply here.

| Form | Example | Notes |
| --- | --- | --- |
| Local file | /scratch/data/granule.h5 | Granule already on disk or on a mounted volume. Fastest, and no credentials needed. |
| GDAL virtual S3 | /vsis3/bucket/key.h5 | Native GDAL form. Honours all AWS_* and CPL_VSIL_CURL_* settings. |
| S3 URL | s3://bucket/key.h5 | Accepted; rewritten internally to /vsis3/bucket/key.h5. |
| HTTPS / CloudFront | https://nisar.asf.earthdatacloud.nasa.gov/... or /vsicurl/https://... | Rewritten internally to /vsicurl/. Out-of-region access; higher latency, and benefits most from the tuning in section 11. |

The file part of the connection string must end in `.h5`; otherwise the driver silently declines to open it and GDAL tries other drivers.

In-region access, meaning an EC2 instance in the same AWS region as the bucket, is dramatically faster than out-of-region access over CloudFront (observed throughout driver development). If you are doing anything at scale, put the compute next to the data.

Path quoting. Historical: early driver versions were sensitive to single versus double slashes and to redundant quoting in S3 paths. The current parser strips one pair of double quotes around the file part and treats :// correctly (section 4.1). If a path still fails for no apparent reason, run gdalinfo /vsis3/bucket/key.h5 without the NISAR: prefix to separate a driver problem from a credentials problem.

## 3.2 Credential resolution

Because the driver relies on GDAL's /vsis3/ layer, S3 credentials are resolved the way GDAL resolves them for every driver, in this order:

1. AWS_ACCESS_KEY_ID / AWS_SECRET_ACCESS_KEY (plus AWS_SESSION_TOKEN for temporary credentials) in the environment.

2. The ~/.aws/credentials and ~/.aws/config files, using AWS_PROFILE / AWS_DEFAULT_PROFILE (or default).

3. EC2 instance / ECS task / IAM role credentials.

The region is not part of the credentials and must be set separately. The NISAR operational buckets are in Oregon:

export AWS_REGION=us-west-2

For a public bucket, disable request signing:

export AWS_NO_SIGN_REQUEST=YES

Partial credential sets break signing: a stale AWS_SESSION_TOKEN left over from a previous profile is a classic cause of H5Fopen failed on a remote file. unset the three variables before switching profiles.

## 3.3 Using an existing AWS profile

The simplest option is to point GDAL at a profile that already exists:

export AWS_PROFILE=saml-pub

export AWS_REGION=us-west-2

gdalinfo NISAR:"$GCOV"

If the profile is backed by SSO or SAML and GDAL cannot refresh it, export temporary credentials into the environment instead. Either use the AWS CLI:

export AWS_REGION=us-west-2

eval $(aws configure export-credentials --format env --profile saml-pub)

or the helper scripts shipped in the repository, which print the same export statements from a profile in ~/.aws/credentials:

eval $(./aws_env.sh saml-pub)

./aws_creds.sh saml-pub      # same, plus os.environ[...] lines for pasting into a notebook

aws configure export-credentials can fail if you already have partial AWS environment variables set, because those take precedence over the profile. Clear them first:

unset AWS_ACCESS_KEY_ID AWS_SECRET_ACCESS_KEY AWS_SESSION_TOKEN

eval $(aws configure export-credentials --format env --profile saml-pub)

JPL-specific (from the original draft, not re-verified): badged JPL users obtain the saml-pub profile through the standard login helpers (aws-login, then bash aws-creds.sh saml-pub), and the ASF DAAC buckets behind nisar.asf.earthdatacloud.nasa.gov need a separate profile created through ondemand-help aws-login-daac.

## 3.4 Jupyter and Python kernels

Notebook kernels do not inherit variables exported in a terminal. Either set AWS_PROFILE and AWS_REGION before starting the kernel, or set the credentials from Python in the first cell. Using boto3 to resolve the profile works for SSO/SAML profiles as well:

import os

import boto3

session = boto3.Session(profile_name="saml-pub")   # may open a browser for SSO login

creds = session.get_credentials().get_frozen_credentials()

os.environ["AWS_ACCESS_KEY_ID"] = creds.access_key

os.environ["AWS_SECRET_ACCESS_KEY"] = creds.secret_key

if creds.token:

os.environ["AWS_SESSION_TOKEN"] = creds.token

os.environ["AWS_REGION"] = session.region_name or "us-west-2"

Section 12 shows how to read data from Python once the environment is set.

# 4. The addressing model: containers, subdatasets, open options

This section is the conceptual core of the guide. Almost every difficulty users hit with the driver comes from confusion about how to name the thing they want to read.

## 4.1 The connection string

A NISAR granule is an HDF5 file holding many rasters, so it does not map onto a single GDAL dataset. The driver therefore behaves like other GDAL subdataset drivers: opening the granule gives you a container that lists what is inside, and you then open one specific raster.

NISAR:<file>[:<hdf5-dataset-path>]

The container form is the file path alone; the subdataset form appends the absolute HDF5 dataset path after a colon:

NISAR:"<path-to-granule.h5>"

NISAR:"<path-to-granule.h5>":<hdf5-dataset-path>

How the driver parses this, from the source:

The NISAR: prefix is removed if present.

The remainder is split at the last colon, unless that colon is immediately followed by //. So NISAR:s3://bucket/x.h5 is a container (the only colon belongs to the URL scheme), and NISAR:s3://bucket/x.h5:/science/LSAR/GCOV/grids/frequencyA/HHHH is a file plus a dataset path.

One pair of double quotes around the file part is stripped, so NISAR:"s3://bucket/x.h5":/science/... is accepted. This is also the form the driver itself emits in SUBDATASET_n_NAME.

The file part must end in .h5.

s3:// is rewritten to /vsis3/; http:// and https:// are rewritten to /vsicurl/.

Opening with update access fails with "The NISAR driver only supports read access."

Quote the whole connection string in the shell, or quote the file part as shown: the : separators and the // in URLs are otherwise easy to mangle.

Use the prefix. For a local .h5 file the prefix is optional: the driver recognises the HDF5 signature and probes for /science/LSAR/identification or /science/SSAR/identification. For a remote file without the prefix the driver only claims the file if the URL contains the string NISAR (it deliberately avoids opening remote files during identification). In either case which driver wins also depends on GDAL's driver order, and GDAL's generic HDF5 driver will open the file without constructing NISAR georeferencing. Always use NISAR: or pass -if NISAR in anything scripted.

## 4.2 Addressing by open option instead of by path

Typing a full HDF5 dataset path is error-prone, so the driver accepts a semantic alternative for the principal rasters: open the container and describe the layer you want with the INST, FREQ and POL open options, and the driver builds the path for you.

These two commands are equivalent, and the second is easier to get right:

gdalinfo NISAR:"$GCOV":/science/LSAR/GCOV/grids/frequencyA/HHHH

gdalinfo -oo FREQ=A -oo POL=HHHH NISAR:"$GCOV"

Setting any of INST, FREQ or POL switches the driver from container mode to single-raster mode; the others take their defaults (LSAR, A, and HHHH for GCOV or HH for everything else). The constructed path is

/science/<INST>/<PRODUCT>/swaths/frequency<F>/<POL>              # L1: RSLC, RIFG, RUNW

/science/<INST>/<PRODUCT>/grids/frequency<F>/<POL>               # L2: GSLC, GCOV, GUNW, GOFF

/science/<INST>/<PRODUCT>/grids/radarData/frequency<F>/<POL>     # L3: SME2

where the product type and level are read from the granule's identification group. POL is validated against the frequency group's listOfCovarianceTerms (GCOV) or listOfPolarizations (everything else); an invalid value fails with Invalid POL open option.

The open-option route therefore works where a raster sits directly under the frequency group: RSLC, GSLC and GCOV, and SME2 where the layer name matches an entry of that list. It does not reach the nested GUNW, GOFF, RIFG and RUNW layers (.../frequencyA/<layerGroup>/<pol>/<layer>); for those, address by full HDF5 path copied from the subdataset list. On GUNW the open-option form fails with The HDF5 dataset '...' does not exist.

How the driver decides what to open, in priority order: an explicit :<hdf5-path> in the connection string → INST/FREQ/POL open options → container mode (no raster bands, SUBDATASETS metadata domain populated).

## 4.3 Instrument selection

NISAR carries two radars, and the HDF5 path begins with the instrument: /science/LSAR/... for the NASA L-band instrument and /science/SSAR/... for the ISRO S-band instrument. The driver detects the instrument automatically by looking for /science/LSAR/identification first and /science/SSAR/identification second, and reports it in the debug log as INST=LSAR or INST=SSAR.

The INST open option only matters when the driver is constructing a path from open options (section 4.2); it must be LSAR or SSAR. When you pass an explicit HDF5 path, the instrument is simply whatever the path says.

gdalinfo -oo INST=SSAR -oo FREQ=A -oo POL=HHHH NISAR:"<s-band-granule.h5>"

S-band granules from ISRO have been read successfully (observed), though some HDF5 link-existence diagnostics were printed on them. Product specification versions also differ between the two instruments, so do not assume identical internal layout.

# 5. Exploring a granule

## 5.1 Listing what is inside

Always start here. Open the container with no other arguments and read the Subdatasets block:

gdalinfo NISAR:"$GCOV"

The driver walks everything under /science/<INST>/ and lists every numeric (non-string) HDF5 dataset. Each entry is a pair. SUBDATASET_n_NAME is a complete, copy-pasteable connection string in the form NISAR:"<file>":<hdf5-path>; SUBDATASET_n_DESC gives [<dims>] <hdf5-path> (<type>), for example [4320x4392] /science/LSAR/GCOV/grids/frequencyA/HHHH (Float32) or [21x720x748] .../radarGrid/incidenceAngle (Float32) for a 3-D cube. Complex rasters are described (complex, Float32). The dimensions are immediately useful, because they tell you whether you are looking at a frequency A grid, a frequency B grid, or a coarse metadata cube, without needing to know the posting conventions.

Granules contain a lot of subdatasets (an S-band GCOV granule has been observed with more than 40), so filter:

gdalinfo NISAR:"$GCOV" | grep HHHH

gdalinfo NISAR:"$GUNW" | grep unwrappedInterferogram

The container's default metadata domain also carries every scalar under /science/<INST>/identification/, keyed by its full path (/science/LSAR/identification/productType, .../boundingPolygon, .../zeroDopplerStartTime, ...):

gdalinfo NISAR:"$GCOV" | grep identification/

## 5.2 Inspecting one raster

Once you have a subdataset name, inspect it directly. This is where you confirm the georeferencing before committing to a warp:

gdalinfo NISAR:"$GCOV":/science/LSAR/GCOV/grids/frequencyA/HHHH

Four things in that output deserve attention. The coordinate system block should name a real projected CRS, typically a UTM zone such as EPSG:32612 for L2 geocoded products (polar stereographic at high latitudes). Origin and Pixel Size should be plausible ground values in metres, not ones and zeros. Corner Coordinates are printed both in projected units and in degrees, and the degrees should place the granule where you expect it. Finally, the Metadata block carries an HDF5_PATH entry echoing which HDF5 dataset you actually opened, which is a useful sanity check when you used open options rather than an explicit path. The same block shows the dataset's own HDF5 attributes (description, units, _FillValue, min_value, ...) and the frequency-group scalars (listOfPolarizations, spacing values, numberOfSubSwaths, ...).

For an L1 raster add -nogcp, otherwise thousands of GCPs bury the rest of the output:

gdalinfo -nogcp NISAR:"$RSLC":/science/LSAR/RSLC/swaths/frequencyA/HH

## 5.3 Metadata

The bulk metadata groups under /science/<INST>/<PRODUCT>/metadata/ are not loaded by default, to keep gdalinfo fast. Retrieving them requires two arguments together, which is a common stumbling block: -oo METADATA= tells the driver which groups to load, and -mdd tells gdalinfo which metadata domains to print. Neither alone gives you the full picture.

gdalinfo -mdd all -oo METADATA=ALL NISAR:"$GCOV"                 # everything

gdalinfo -mdd NISAR_ORBIT -oo METADATA=ORBIT NISAR:"$GCOV"       # one group

gdalinfo -mdd all -oo METADATA=ORBIT,RADARGRID NISAR:"$GSLC"     # a comma list

Each requested group appears as a domain named NISAR_<GROUP>. The metadata domains you will see:

| Domain | Content |
| --- | --- |
| default | Container: root attributes plus the identification scalars keyed by full path. Raster: the dataset's HDF5 attributes plus frequency-group scalars. |
| NISAR_GLOBAL | Root-level HDF5 attributes (CF-style discovery attributes such as title, institution, reference_document). |
| SUBDATASETS | Container only. |
| DERIVED_SUBDATASETS | Numeric rasters only; see section 10. |
| NISAR_ATTITUDE, NISAR_CALIBRATIONINFORMATION, NISAR_CEOSANALYSISREADYDATA, NISAR_ORBIT, NISAR_PROCESSINGINFORMATION, NISAR_RADARGRID, NISAR_SOURCEDATA | Only when requested with METADATA=. |

Useful items surfaced this way include the identification block (granuleId, productType, productVersion, productSpecificationVersion), the processing information including the ISCE3 software version that generated the product, and the pointer to the companion static-layers granule (section 7.7).

## 5.4 Statistics: know what -stats actually returns

The driver overrides GetStatistics(), GetMinimum() and GetMaximum() to avoid scanning a remote raster when the producer already wrote the answer. If the HDF5 dataset carries min_value/max_value (or valid_min/valid_max) attributes, or the caller allows approximation (gdalinfo -approx_stats), the driver returns those attribute values without touching pixels. Mean and standard deviation come from mean_value/sample_stddev attributes if present; otherwise they are synthesised as (min+max)/2 and (max-min)/6. With no attributes at all, -approx_stats falls back to 0 and 5.

Only gdalinfo -stats on a band without min/max attributes triggers a real pixel scan (which then does honour the mask band if MASK=YES):

gdalinfo -stats -oo FREQ=A -oo POL=VVVV NISAR:"$GCOV"

If you need measured statistics, translate to GeoTIFF first and run gdalinfo -stats on that, or compute them in NumPy. Do not quote driver-reported mean or standard deviation in a report unless you have confirmed the dataset carries mean_value/sample_stddev.

## 5.5 Point queries

To read a single pixel value without extracting anything, use gdallocationinfo with pixel and line coordinates, or with a geographic coordinate on a geocoded product:

gdallocationinfo NISAR:"$SME2":/science/LSAR/SME2/grids/radarData/frequencyA/sigma0HH 800 800

gdallocationinfo -wgs84 NISAR:"$GSLC":/science/LSAR/GSLC/grids/frequencyA/HH -118.25 34.05

Because granules use HDF5 paged aggregation with a 4 MiB page, a single-pixel read on a remote file still costs at least one page (observed); it is fast, but not free.

## 5.6 Debugging with CPL_DEBUG

When the driver does something you do not expect, turn on GDAL's debug output. CPL_DEBUG=NISAR_DRIVER shows only the driver's messages; CPL_DEBUG=ON shows everything, including the HTTP requests GDAL makes:

CPL_DEBUG=NISAR_DRIVER gdalinfo -oo FREQ=A -oo POL=HHHH NISAR:"$GCOV"

The log tells you which product it identified and at what level, whether it routed I/O through the VSI layer, where it found coordinate arrays for the GeoTransform, which EPSG code it read, and how many GCPs it set. Lines look like this:

NISAR_DRIVER: Identified Product: INST=LSAR, Type=GCOV, Level=L2 (L1=0, L2=1, L3=0)

NISAR_DRIVER: Remote cloud file detected. Routing HDF5 through GDAL VSIL VFL...

NISAR_DRIVER: Set HDF5 FAPL page buffer to 4 MiB.

NISAR_DRIVER: GetGeoTransform: Found coordinate arrays at: /science/LSAR/GCOV/grids/frequencyA

NISAR_DRIVER: GetGeoTransform: Derived from coordinates. Origin=(...) Res=(...)

NISAR_DRIVER: Successfully imported EPSG:32612.

## 5.7 The granule footprint

The bounding polygon lives at /science/LSAR/identification/boundingPolygon as an HDF5 string scalar, not a raster. The driver builds GDAL raster bands from numeric arrays only, so string datasets are skipped by the subdataset listing and cannot be opened as a dataset.

The driver does read the value into the container's default metadata domain, keyed by its full path, so retrieve it as a metadata item:

gdalinfo NISAR:"$GCOV" | grep boundingPolygon

from osgeo import gdal

ds = gdal.Open('NISAR:granule.h5')

wkt = ds.GetMetadataItem('/science/LSAR/identification/boundingPolygon')

The driver is a raster driver only; there is no OGR vector layer for footprints, so ogrinfo and ogr2ogr do not apply.

# 6. Open option reference

Open options are passed as -oo KEY=VALUE and may be repeated; every GDAL utility accepts them, and gdal.OpenEx(..., open_options=[...]) takes them in Python. They are registered in the driver's DMD_OPENOPTIONLIST, so gdalinfo --format NISAR prints the authoritative list for the version you are running. GDAL matches option names case-insensitively; the canonical spelling is upper case and that is what this guide uses. (Historical: earlier release notes wrote freq and pol in lower case. Both spellings work.)

## 6.1 All options

| Option | Values | Default | Meaning |
| --- | --- | --- | --- |
| INST | LSAR, SSAR | LSAR | Instrument group, used only when constructing a path from open options. |
| FREQ | A, B | A | Frequency sub-band. Frequency B is absent from many granules and has a coarser posting. |
| POL | product-dependent, see 6.2 | HHHH for GCOV, HH otherwise | Polarization or covariance term. Validated against the granule's listOfCovarianceTerms / listOfPolarizations. Setting any of INST, FREQ, POL opens that single raster instead of the container. |
| METADATA | ALL or a comma list of ATTITUDE, CALIBRATIONINFORMATION, CEOSANALYSISREADYDATA, ORBIT, PROCESSINGINFORMATION, RADARGRID, SOURCEDATA | none | Loads the named /metadata/<group> trees into NISAR_<GROUP> metadata domains. Pair with gdalinfo -mdd. |
| MASK | YES, NO | NO | Expose the product validity mask as the GDAL mask band. See section 9. |
| DEM_FILE | path or /vsis3/... URL | none | DEM for 3-D metadata cube interpolation. Required when QUANTITY is set; has no effect otherwise. |
| DEM_RESAMPLING | NEAREST, BILINEAR, CUBIC, CUBICSPLINE | CUBICSPLINE | Resampling used when warping the DEM onto the target grid. |
| QUANTITY | any string | none | Its presence routes the open to the cube-interpolation dataset (section 7.6). The value itself is not used to locate the cube; the connection string must point at it. |
| ENABLE_PAGE_BUFFERING | YES, NO | NO | Reserved. The driver always uses a 4 MiB HDF5 page buffer; this option has no other effect today. |

There are no `LAYER` or `MEASURE` options. Earlier drafts of this guide described them; they do not exist in the driver. GUNW, GOFF, RIFG and RUNW layers are addressed by full HDF5 path (section 7.4).

## 6.2 Polarization values by product

GCOV holds covariance-matrix terms, so its POL values are four characters: the diagonal terms HHHH, HVHV, VHVH and VVVV are real-valued, while off-diagonal terms such as HHHV are complex. Products that hold a single-polarization signal, including RSLC, GSLC and GUNW, use two-character values such as HH, HV, VH and VV. Compact-polarimetric and S-band granules introduce further values such as RHRH, so list the granule rather than guessing; the driver will tell you when a value is not in the granule's list.

# 7. Per-product recipes

Paths below follow the driver's own construction rules and the NISAR product specifications; the concrete layer names inside them must be confirmed against the subdataset list of the granule in front of you.

## 7.1 RSLC, RIFG and RUNW (Level 1)

Level 1 products are in radar geometry: range and azimuth, not a map projection. Their rasters live under /science/<INST>/<PRODUCT>/swaths/frequency{A,B}/... (note swaths, where the geocoded products use grids). The driver attaches Ground Control Points built from metadata/geolocationGrid rather than an affine GeoTransform, and everything about handling L1 follows from that. GetGeoTransform() deliberately fails for L1, so tools that insist on an affine transform will complain.

List the swath rasters, then open one. Adding -nogcp suppresses the GCP listing, which is otherwise long enough to bury the rest of the output:

gdalinfo NISAR:"$RSLC"

gdalinfo -nogcp NISAR:"$RSLC":/science/LSAR/RSLC/swaths/frequencyA/HH

gdalinfo -nogcp -oo FREQ=A -oo POL=HH NISAR:"$RSLC"      # same raster via open options

Drop -nogcp when you actually want to examine the GCPs, for instance to check their spacing before a warp. RSLC data are complex, so the derived subdatasets of section 10 apply.

GCP generation reads startingRange, slantRangeSpacing and nominalAcquisitionPRF from swaths/frequencyA regardless of which raster you opened (inference from source, not observed), so treat frequency-B L1 georeferencing with suspicion and verify it independently.

There is no mask dataset alongside L1 rasters, so MASK=YES yields GDAL's all-valid mask. (Earlier drafts stated that RSLC validity is derived on the fly from acquisition start/stop vectors; the current driver does not do this.)

## 7.2 GSLC (Level 2)

GSLC is geocoded complex imagery. It behaves like GCOV for addressing purposes, with two-character polarization values, and like GCOV it carries a real projected CRS and a proper GeoTransform:

gdalinfo NISAR:"$GSLC"

gdalinfo -oo FREQ=A -oo POL=HH NISAR:"$GSLC"

Because the pixels are complex, the derived subdatasets described in section 10 are particularly useful here: they let you get amplitude, phase or intensity without writing any code.

One current limitation: metadata cube interpolation (section 7.6) hard-codes the GCOV and GUNW reference grids. If a cube interpolation command works on GCOV but fails on GSLC with "Unsupported product type", this is why; it is a driver gap rather than a mistake on your part.

## 7.3 GCOV (Level 2)

GCOV is the best-exercised product and a good place to build intuition. The principal rasters are the covariance terms under grids:

gdal_translate -oo FREQ=A -oo POL=HHHH NISAR:"$GCOV" gcov_hhhh.tif

There is also a mask raster you can read as a layer in its own right, which is worth doing when you want to understand why pixels are being dropped. Opened directly, the driver gives it NoData 255, a colour table and category names (Invalid or partially focused, Valid (Sub-swath 1) ... Valid (Sub-swath 5)):

gdalinfo NISAR:"$GCOV":/science/LSAR/GCOV/grids/frequencyA/mask

### Noise equivalent backscatter and other calibration grids

Noise equivalent backscatter is not one of the standard grid rasters. It sits on a coarser calibration grid under metadata/calibrationInformation, with its own xCoordinates/yCoordinates vectors and projection dataset. The driver's GeoTransform lookup walks up from the opened dataset until it finds a sibling xCoordinates, and it does this for any path containing /grids/, /calibrationInformation/ or /radarGrid/, so these rasters come back georeferenced.

Two things to internalise. First, the calibration grid and the principal grid have deliberately different corner coordinates and extents; seeing different corners between the two is expected and is not evidence of a bug. Second, a pixel size near 1 unit means the GeoTransform was not found (historical: this was the failure mode before calibration grids were handled), and warping such a raster produces a tiny square in the wrong place on the map. Inspect before warping:

gdalinfo NISAR:"$GCOV":\

/science/LSAR/GCOV/metadata/calibrationInformation/frequencyA/noiseEquivalentBackscatter/HH \

| grep -A1 "Pixel Size"

## 7.4 GUNW and GOFF (Level 2)

GUNW and GOFF are geocoded, so they already have a projected CRS and need no GCP handling. Their complication is depth: layers nest under a layer group and then a polarization, grids/frequencyA/<layerGroup>/<pol>/<layer>. Georeferencing is resolved by walking up to the frequency group, so it works at any depth, but FREQ/POL cannot express these paths. Address them by full HDF5 path, copied from the subdataset list:

gdalinfo NISAR:"$GUNW" | grep -E "unwrappedInterferogram|pixelOffsets"

Layer names below follow the GUNW product specification; confirm the spelling against your granule (an early announcement mis-spelled coherenceMagnitude, which is exactly why copying from gdalinfo is safer):

# unwrapped phase

gdalinfo NISAR:"$GUNW":\

/science/LSAR/GUNW/grids/frequencyA/unwrappedInterferogram/HH/unwrappedPhase

# coherence and connected components from the same granule

gdal_translate \

NISAR:"$GUNW":/science/LSAR/GUNW/grids/frequencyA/unwrappedInterferogram/HH/coherenceMagnitude \

gunw_coherence.tif

gdal_translate \

NISAR:"$GUNW":/science/LSAR/GUNW/grids/frequencyA/unwrappedInterferogram/HH/connectedComponents \

gunw_concomp.tif

# pixel offsets live in their own layer group

gdal_translate \

NISAR:"$GUNW":/science/LSAR/GUNW/grids/frequencyA/pixelOffsets/HH/alongTrackOffset \

gunw_along_track_offset.tif

Other GUNW layers you will typically find under unwrappedInterferogram/<pol>/ include ionospherePhaseScreen, ionospherePhaseScreenUncertainty and mask; under pixelOffsets/<pol>/, slantRangeOffset and correlationSurfacePeak; and a wrappedInterferogram group holding the complex wrapped interferogram. GOFF holds the pixelOffsets layers only. Connected components are integer-valued; the wrapped interferogram is complex.

## 7.5 SME2 (Level 3)

SME2 soil-moisture rasters are read the same way, under a radarData group:

gdalinfo NISAR:"$SME2"

gdallocationinfo NISAR:"$SME2":/science/LSAR/SME2/grids/radarData/frequencyA/sigma0HH 800 800

The driver applies the same L2 georeferencing logic (coordinate vectors and projection dataset found by walking up the path) to L3 products. SME2 is the least exercised product: a warning reading "Unknown NISAR product structure. Georeferencing may be absent." was observed on an earlier driver version while the returned pixel values were correct, and that message is no longer present in the current source. Treat the lesson as still valid: verify the CRS and GeoTransform before you warp or overlay SME2 output.

## 7.6 Radar-grid metadata cubes

Quantities such as incidence angle, look angle and the line-of-sight or along-track unit vector components are stored under metadata/radarGrid/ as coarse three-dimensional cubes: height × y × x. The driver exposes a 3-D dataset as a multi-band raster, one band per height level, with a GeoTransform from the cube's own coordinate vectors. When a sibling heightAboveEllipsoid dataset exists and has one entry per band, each band is described Height: <z> m and carries HEIGHT_METERS and Z_VALUE band metadata.

gdalinfo NISAR:"$GCOV":/science/LSAR/GCOV/metadata/radarGrid/incidenceAngle

Expect a coarse pixel size (500 m in one observed case) and a band count matching the number of height levels (21 in that same case). Select a single height level with the standard GDAL band argument:

gdal_translate -b 5 -of GTiff \

NISAR:"$GCOV":/science/LSAR/GCOV/metadata/radarGrid/incidenceAngle \

cube_band5.tif

One cosmetic issue: the cube's dataset metadata is reported for every band, so the same values repeat once per band in gdalinfo output.

### Interpolating a cube onto the imaging grid

Selecting a single height level is rarely what you actually want. The physically correct operation is to interpolate the cube in three dimensions, horizontally onto the imaging grid and vertically using the terrain height at each pixel, so that the result lands on the same grid as the imagery. The driver implements this when QUANTITY and DEM_FILE are both given:

gdal_translate -co TILED=YES -co BLOCKXSIZE=512 -co BLOCKYSIZE=512 -co COMPRESS=ZSTD \

-oo QUANTITY=incidenceAngle -oo DEM_FILE="$DEM" -oo DEM_RESAMPLING=CUBICSPLINE \

NISAR:"$GCOV":/science/LSAR/GCOV/metadata/radarGrid/incidenceAngle \

incidence_angle.tif

What happens, from the source (v0.6.6):

1. The presence of QUANTITY routes the open to the interpolation dataset. If DEM_FILE is missing the open fails with "DEM_FILE open option is REQUIRED when QUANTITY is specified."

2. The cube is whatever dataset the connection string points at. The QUANTITY value is not used to locate it, so pass the cube path explicitly.

3. The target grid is chosen by substring match on the connection string: a string containing GCOV uses /science/LSAR/GCOV/grids/frequencyA/HHHH; one containing GUNW uses /science/LSAR/GUNW/grids/frequencyA/unwrappedInterferogram/HH/unwrappedPhase; anything else fails with "Unsupported product type. Cannot determine reference grid." LSAR, frequency A and those two layers are hard-coded, so GSLC, SSAR, frequency B and Level 1 are not supported.

4. The DEM is warped onto the target grid with DEM_RESAMPLING, the whole cube is loaded into memory, and each output pixel is interpolated in height. The output is a single-band Float32 raster with the target grid's georeferencing, 512×512 blocks, and NaN where the DEM or cube has no value.

A global public DEM VRT is available in both S3 and HTTPS form (observed); its resolution need not match NISAR posting, which is why DEM_RESAMPLING exists:

# S3:    /vsis3/sds-n-cumulus-prod-nisar-products/DEM/v1.2/EPSG4326/EPSG4326.vrt

# HTTPS: https://nisar.asf.earthdatacloud.nasa.gov/NISAR/DEM/v1.2/EPSG4326/EPSG4326.vrt

Passing DEM_FILE to gdalinfo without QUANTITY does nothing: the option is only read on the interpolation path. (Earlier drafts stated otherwise.)

## 7.7 Static layers

Static layers ship as a separate companion granule rather than inside the product. Its URL is carried in the granule metadata (look for a staticLayersDataAccess-style item in the identification or processing-information domains, section 5.3); retrieve it and then open that granule as a NISAR product in its own right.

# 8. Reprojection and warping to latitude-longitude

This is the most requested operation, and the answer depends entirely on whether your product is geocoded.

## 8.1 Level 2 and Level 3, the straightforward case

GCOV, GSLC, GUNW, GOFF and SME2 are already geocoded, typically into a UTM zone. Reprojecting to geographic coordinates is therefore an ordinary CRS transformation, and gdalwarp handles it with no NISAR-specific arguments beyond the open options or path that select your layer. The GeoTransform is derived from the product's xCoordinates/yCoordinates (pixel centres, shifted half a pixel to GDAL's corner convention) and the CRS from the sibling projection dataset's epsg_code attribute (or its WKT).

A complete, production-quality command for GUNW unwrapped phase:

gdalwarp -t_srs EPSG:4326 -r near -of GTiff \

-srcnodata nan -dstnodata nan \

-co COMPRESS=DEFLATE -co PREDICTOR=3 -co TILED=YES -co BIGTIFF=IF_SAFER \

-multi -wo NUM_THREADS=ALL_CPUS \

NISAR:"$GUNW":/science/LSAR/GUNW/grids/frequencyA/unwrappedInterferogram/HH/unwrappedPhase \

gunw_unwrapped_phase_ll.tif

The same pattern for a GCOV backscatter term, using open options:

gdalwarp -t_srs EPSG:4326 -r bilinear -of GTiff \

-oo FREQ=A -oo POL=HHHH \

-co COMPRESS=DEFLATE -co PREDICTOR=3 -co TILED=YES \

-multi -wo NUM_THREADS=ALL_CPUS \

NISAR:"$GCOV" gcov_hhhh_ll.tif

## 8.2 Level 1, and why you should be careful

RSLC carries GCPs rather than a GeoTransform, so warping it means fitting a transformation to those GCPs. GDAL will do this, and the result looks convincing, but it is an approximation to a geometry that is properly handled by geocoding with a DEM. Use it for quick looks and for demonstrating that the GCPs exist, not for accurate geolocation.

# highest accuracy from the GCPs: thin-plate spline

gdalwarp -t_srs EPSG:4326 -tps -r cubic \

NISAR:"$RSLC":/science/LSAR/RSLC/swaths/frequencyA/HH RSLC_HH_tps.tif

# fast preview: second-order polynomial, cropped to an extent in degrees

gdalwarp -t_srs EPSG:4326 -order 2 -r near -co COMPRESS=LZW \

-te -62.424 -8.862 -62.299 -8.761 \

NISAR:"$RSLC":/science/LSAR/RSLC/swaths/frequencyA/HH warped-cropped-rslc.tif

If you need geolocated Level 1 imagery for analysis, use the geocoded L2 products, which were produced with the full geometry and a DEM.

## 8.3 Choosing a resampling method

The default resampling in gdalwarp is nearest neighbour, and for most NISAR layers that is also the right choice. The table below gives the reasoning, which is worth understanding rather than memorising.

| Layer type | Method | Why |
| --- | --- | --- |
| Unwrapped phase | -r near | Piecewise continuous across connected-component boundaries. Interpolation invents values straddling those discontinuities and across decorrelated edges. |
| Connected components | -r near | Integer labels. Any averaging produces meaningless intermediate labels. |
| Wrapped phase, complex data | -r near | Interpolating a wrapped quantity across the branch cut is wrong. Resample real and imaginary parts separately if you must interpolate. |
| Backscatter (GCOV) | -r bilinear | Continuous positive quantity. Bilinear or cubic is defensible; consider -r average when downsampling. |
| Coherence | -r bilinear | Continuous and bounded. Interpolation is acceptable. |
| Masks, flags | -r near | Categorical by definition. |
| Incidence angle, look angle | -r bilinear | Smooth geometric field. |

If you want a visually smooth phase product, do not interpolate the phase. Warp connected components alongside it with nearest neighbour and mask afterwards.

## 8.4 Extent, resolution and alignment

To crop while warping, give a target extent in the units of the target CRS, which means degrees when warping to EPSG:4326:

-te <xmin> <ymin> <xmax> <ymax>

Output resolution is set with -tr, again in target CRS units. It is usually better to omit it and let gdalwarp derive a resolution from the source, because hand-specifying degrees to match a metre posting is easy to get wrong. When you do need a specific grid, -tap aligns the output to whole multiples of the pixel size, which makes tiles from different granules line up:

-tr 0.0008 0.0008 -tap

Nodata handling matters for phase and offset layers, which use NaN as fill. Set both source and destination nodata so that fill is not warped into your valid data:

-srcnodata nan -dstnodata nan

# 9. Masking

## 9.1 What the mask means, by product

With -oo MASK=YES the driver implements GDAL's mask-band concept: it looks for a sibling dataset named mask in the opened raster's parent group (grids/frequencyA/mask for GCOV and GSLC, grids/frequencyA/unwrappedInterferogram/HH/mask for GUNW unwrapped layers) and exposes it as a per-dataset mask band (GMF_PER_DATASET). If there is no such dataset you get GDAL's all-valid mask. The mapping from stored values to valid or invalid, from the source:

| Product | Mask source and interpretation |
| --- | --- |
| GUNW | Stored byte v in 1–254 is decoded as reference sub-swath (v/10)%10 and secondary sub-swath v%10; the pixel is valid only if both are non-zero. 0 and 255 are invalid. |
| Everything else (GCOV, GSLC, ...) | Stored mask raster. Values 1 through 5 (sub-swath number) are valid; all other values, including 0 and 255, are invalid. |
| L1 (RSLC, RIFG, RUNW) | No mask dataset next to the raster; MASK=YES yields an all-valid mask. |

## 9.2 The default is NO; pin it

Masking is off by default. gdal_translate and gdalwarp therefore copy stored fill values unless you set MASK=YES. Any pipeline you inherited, and any comparison across driver versions, needs the mask state pinned explicitly rather than left to the default:

-oo MASK=YES    # apply the validity mask

-oo MASK=NO     # keep all stored values, including masked-out pixels

Historical: masking was applied by default in versions 0.1.4 through 0.2.x and made opt-in in 0.3.0, because applying it introduced visible artifacts in some products and not applying it is noticeably faster for gdal_translate. The same command run on driver versions either side of 0.3.0 returns different pixel values.

Keeping masked values is often what you actually want in analysis, for example when computing histograms where excluded pixels would bias the distribution. Quantifying the effect with gdalinfo -stats only works on a band that has no min_value/max_value attributes (section 5.4); otherwise the driver returns the producer's attributes regardless of MASK. (Observed on an earlier version: one GCOV case showed the mean moving from 0.075 to 0.083 and the standard deviation from 1.438 to 1.539 between MASK=NO and MASK=YES, with the maximum unchanged.)

## 9.3 Carrying the mask into your output

A GeoTIFF has no equivalent of a GDAL mask band unless you create one. With gdalwarp, add -dstalpha to write the mask as an alpha band, which preserves it for downstream tools and for display:

gdalwarp -of GTiff -dstalpha \

-oo FREQ=A -oo POL=HVHV -oo MASK=YES \

NISAR:"$GCOV" masked_output_HVHV.tif

gdal_translate copies the mask as an internal TIFF mask by default (-mask auto). Derived subdatasets (section 10) do not inherit the mask.

# 10. Derived subdatasets and virtual overviews

## 10.1 Derived subdatasets

For every numeric raster the driver advertises GDAL derived subdatasets in the DERIVED_SUBDATASETS metadata domain: virtual datasets that apply a pixel function on read. Nothing is precomputed and nothing is stored, so this costs you only the read you were doing anyway.

Complex rasters (GSLC, RSLC, off-diagonal GCOV terms, wrapped GUNW interferograms) advertise AMPLITUDE, PHASE, REAL, IMAG, INTENSITY and CONJ.

Real-valued rasters advertise LOGAMPLITUDE (log10 of amplitude), useful for backscatter spanning many orders of magnitude.

The advertised names are complete connection strings you can pass straight to another GDAL command:

DERIVED_SUBDATASET:AMPLITUDE:"NISAR:<granule.h5>:<hdf5-path>"

Quote the whole thing as a single shell argument:

gdal_translate \

"DERIVED_SUBDATASET:AMPLITUDE:NISAR:$GSLC:/science/LSAR/GSLC/grids/frequencyA/HH" \

amplitude.tif

gdal_translate \

"DERIVED_SUBDATASET:PHASE:NISAR:$GSLC:/science/LSAR/GSLC/grids/frequencyA/HH" \

phase.tif

gdal_translate \

"DERIVED_SUBDATASET:LOGAMPLITUDE:NISAR:$GCOV:/science/LSAR/GCOV/grids/frequencyA/HHHH" \

hhhh_db.tif

A derived subdataset is an ordinary GDAL dataset, but the driver's mask band is attached to the underlying raster, not to the derived view; do not assume a derived subdataset inherits masking.

## 10.2 Virtual overviews

Every band exposes virtual power-of-two overviews (2×, 4×, ... up to NISAR_MAX_VIRTUAL_OVR, default 16), computed by decimated reads of the full-resolution chunks; nothing is stored. This is enough for QGIS and tile servers to zoom out without pre-built pyramids. The cost is proportional to the number of chunks touched, so if zoomed-out views of a very large product are slow, lower NISAR_MAX_VIRTUAL_OVR (or set it to 1 to disable), and for repeated analysis convert to a Cloud-Optimized GeoTIFF instead:

gdal_translate -of COG -co COMPRESS=DEFLATE \

NISAR:"$GCOV":/science/LSAR/GCOV/grids/frequencyA/HHHH HHHH_cog.tif

# 11. Performance

## 11.1 How the driver reads

Four facts explain most of the performance behaviour you will observe:

GDAL block size = HDF5 chunk size. Every IReadBlock maps onto exactly one HDF5 chunk, so no chunk is read twice. Read windows aligned to the chunk grid are fastest. Chunk offsets are mapped lazily on first read.

Mega-fetch. When a block is missing from the cache, the driver reads a square grid of NISAR_PREFETCH_GRID × NISAR_PREFETCH_GRID neighbouring chunks in one contiguous range request (capped by NISAR_MAX_MEGAFETCH_BYTES), decompresses them in parallel (GDAL_NUM_THREADS) and pushes all of them into the GDAL block cache.

Fixed HDF5 tuning. Files are opened with a 4 MiB HDF5 page buffer, matching the 4 MiB paged-aggregation page NISAR granules are written with, and each dataset gets an 8 MiB / 521-slot chunk cache. Neither is configurable today.

Retries. GDAL_HTTP_MAX_RETRY is set to 5 by the driver if you have not set it, so transient S3 errors do not fail the read.

## 11.2 Environment variables

These are GDAL configuration options: set them as environment variables, with --config NAME VALUE on the command line, or with gdal.SetConfigOption() in Python.

| Variable | Default | Effect |
| --- | --- | --- |
| NISAR_PREFETCH_GRID | 1 | Side of the N×N grid of HDF5 chunks coalesced into one range request on a cache miss. 1 fetches only the requested chunk (best for tile servers, QGIS and small windows). Larger values such as 24 coalesce many chunks into one large read, much faster for full-scene batch processing. |
| NISAR_MAX_MEGAFETCH_BYTES | 16777216 (16 MiB) | Upper bound on one coalesced read, so a large NISAR_PREFETCH_GRID cannot produce requests too big for the network or memory. |
| NISAR_MAX_VIRTUAL_OVR | 16 | Largest decimation factor for which a virtual overview is synthesised. 1 disables virtual overviews. |
| NISAR_EXPORT_ZARR | NO | When YES, writes a Kerchunk-style JSON sidecar under /tmp/ describing the HDF5 chunk map of the opened raster, for Zarr / xarray tooling. Debug and interoperability only. |
| GDAL_NUM_THREADS | GDAL default | Threads used to decompress chunks in parallel. ALL_CPUS is reasonable. |
| GDAL_CACHEMAX | GDAL default | GDAL block cache size in MB. Raise it (e.g. 2048) when repeatedly reading large remote rasters. |
| GDAL_HTTP_MAX_RETRY | 5 (set by driver) | Retries on failed HTTP range requests. |
| GDAL_DISABLE_READDIR_ON_OPEN | — | EMPTY_DIR stops GDAL listing the containing S3 prefix on open, which is pure latency when you already know the exact key. |
| GDAL_PAM_ENABLED | — | NO stops GDAL writing .aux.xml sidecars next to local granules, which is harmless but clutters directories and fails on read-only media. |

A reasonable starting point for full-scene batch work from S3:

export NISAR_PREFETCH_GRID=24

export GDAL_CACHEMAX=2048

export GDAL_NUM_THREADS=ALL_CPUS

export GDAL_DISABLE_READDIR_ON_OPEN=EMPTY_DIR

export GDAL_PAM_ENABLED=NO

For interactive or tiled access (QGIS, TiTiler) leave NISAR_PREFETCH_GRID at its default of 1. Standard GDAL /vsis3/ and /vsicurl/ options (AWS_*, CPL_VSIL_CURL_*, GDAL_HTTP_*, VSI_CACHE, ...) also apply.

Variables from older notes such as NISAR_CHUNK_CACHE_SIZE_MB do not exist in the current driver and are silently ignored.

## 11.3 What to expect (observed, not benchmarks)

These figures come from development tuning runs on specific granules and instances; they show the shape of the behaviour, not guaranteed numbers.

NISAR granules use HDF5 paged aggregation with a 4 MiB page, so a single-pixel read still transfers at least one page; there is a floor on the cost of any remote read. Reading a full 1.1 GB GCOV raster took on the order of 150 range requests.

Peak memory for a full-raster read was about 1.1 GiB for the driver, against roughly 24 GiB for an h5py script and 28 GiB for xarray that used a page buffer large enough to hold entire layers. For time-series work, where you read many granules concurrently or accumulate statistics block by block, this bounded footprint is decisive; build workflows around block-wise reads.

On full-raster reads the driver was roughly twice as fast as Cloud-Optimized GeoTIFF of the same data (about 12 range requests against 40–50, at the cost of higher transfer volume), and around five times faster than h5py. On very small windows (a tenth of a GUNW raster, involving very few chunks) fixed overheads dominate and h5py compared favourably.

Optimal settings depend on product type, window size, data type, and whether you are in-region or out-of-region. A single tuning configuration is not optimal everywhere.

# 12. Python (osgeo.gdal)

Everything above is available from Python through the GDAL bindings; the connection strings and open options are identical.

from osgeo import gdal

gdal.UseExceptions()

# Container: list the rasters in the product

container = gdal.Open("NISAR:L2_GCOV.h5")

for name, desc in container.GetSubDatasets():

print(name, "-", desc)

# Open one raster with open options and read a window

ds = gdal.OpenEx(

"NISAR:L2_GCOV.h5",

open_options=["FREQ=A", "POL=HHHH", "MASK=YES", "METADATA=ORBIT"],

)

band = ds.GetRasterBand(1)

window = band.ReadAsArray(xoff=0, yoff=0, win_xsize=1024, win_ysize=1024)

print(window.shape, band.GetNoDataValue(), ds.GetGeoTransform())

print(ds.GetMetadata("NISAR_ORBIT"))

# Open by full path on S3

ds = gdal.Open("NISAR:s3://my-bucket/path/to/file.h5:/science/LSAR/RSLC/swaths/frequencyA/HH")

print(ds.RasterXSize, ds.RasterYSize, gdal.GetDataTypeName(ds.GetRasterBand(1).DataType))

Read in chunk-aligned windows rather than calling ReadAsArray() on a whole remote raster, and set GDAL_CACHEMAX / NISAR_PREFETCH_GRID with gdal.SetConfigOption() before the first open.

# 13. Troubleshooting

| Symptom | Cause / fix |
| --- | --- |
| gdalinfo --formats does not list NISAR | The plugin is not on GDAL's plugin path. Check that gdal-driver-nisar is installed in the active environment and that $CONDA_PREFIX/lib/gdalplugins/gdal_NISAR.* exists. If GDAL comes from outside conda, set GDAL_DRIVER_PATH. |
| gdalinfo file.h5 opens with the HDF5 driver instead of NISAR | Prefix the path with NISAR: (or pass -if NISAR). |
| H5Fopen failed for '...' on a remote file | Usually credentials or region. Test the URL directly with gdalinfo /vsis3/bucket/key.h5, check AWS_REGION and AWS_PROFILE, unset stale AWS_* variables, or run aws s3 ls s3://bucket/key.h5 --profile <profile>. Add --debug on to see the HTTP requests GDAL makes. |
| The driver silently declines and another driver (or "not recognized") appears | The file part of the connection string does not end in .h5, or a remote URL without the NISAR: prefix does not contain NISAR. |
| The HDF5 dataset '...' does not exist | The path (typed or constructed from FREQ/POL) is not in this granule. List the container and copy the path; for GUNW/GOFF/RIFG/RUNW use the full path, not open options. |
| Invalid INST open option, Invalid FREQ open option, Invalid POL open option: '...' | Check INST (LSAR/SSAR), FREQ (A/B) and POL against the granule's listOfPolarizations / listOfCovarianceTerms. |
| DEM_FILE open option is REQUIRED when QUANTITY is specified | Interpolation mode needs both QUANTITY and DEM_FILE. |
| Unsupported product type. Cannot determine reference grid. | Cube interpolation is hard-coded for GCOV and GUNW connection strings (LSAR, frequency A). |
| Pixel size near 1, origin near 0 | No GeoTransform was found for this dataset. For L1 that is expected (use the GCPs); for L2/L3 check the driver version and the path. |
| Warped output is a tiny square in the wrong place | Same cause as above; you warped a raster with an identity GeoTransform. |
| Statistics look implausible or identical with and without MASK | -stats/-approx_stats returned producer attributes or synthesised values (section 5.4). Translate to GeoTIFF and compute there. |
| Slow full-scene reads from S3 | Increase NISAR_PREFETCH_GRID (e.g. 24) and GDAL_CACHEMAX; make sure GDAL_NUM_THREADS is not 1; compute in-region. |
| GLIBC_2.xx not found on every GDAL command | The Linux plugin binary is newer than the host libc; needs a rebuild for your platform. |
| conda cannot solve the environment | GDAL minor-version mismatch with the plugin's pin. Install into a fresh environment. |

Enable driver debug output with CPL_DEBUG=NISAR_DRIVER (or CPL_DEBUG=ON for everything).

# 14. Version history and behaviour changes

Because several releases changed output rather than only fixing crashes, knowing your version is part of knowing your data. The entries for versions before 0.6.6 are taken from release announcements and field notes and have not been re-verified; the row for 0.6.6 summarises what this edition verified in source.

| Version | Change | Affects results |
| --- | --- | --- |
| 0.1.1 | Rebuild addressing the GLIBC dependency error on older Linux hosts. | no |
| 0.1.3 | Changes to metadata output. | no |
| 0.1.4 | MASK open option introduced, applied by default. | yes |
| 0.1.7 | GeoTransform fixed for GUNW principal rasters. Calibration-grid projection metadata handled, addressing the NEB georeferencing problem. | yes |
| 0.1.8 | Radar-grid metadata cubes interpreted as multi-band rasters with a correct GeoTransform; band selection with -b. | yes |
| 0.1.9 | DRIVER_VERSION with build date reported via gdalinfo --format NISAR. | no |
| 0.3.0 | Path quoting and slash handling reworked. Mask no longer applied by default. Remote reads through HDF5's ROS3 driver with AWS-style credential sourcing. | yes |
| 0.6.6 (current) | Built against GDAL 3.12. All remote I/O routed through GDAL VSI via a custom HDF5 Virtual File Layer (ROS3 no longer used; standard GDAL AWS_* configuration applies). Open options registered in DMD_OPENOPTIONLIST; DEM_RESAMPLING added; no LAYER/MEASURE options. Chunk-aligned mega-fetch reads with NISAR_PREFETCH_GRID / NISAR_MAX_MEGAFETCH_BYTES, virtual overviews, attribute-based statistics, GUNW-specific mask decoding, optional Kerchunk sidecar. | yes (statistics, masks) |

# 15. Verification checklist

Before you use driver output for anything quantitative, work through this list. Each item corresponds to a failure that has actually occurred.

Confirm the driver version from the binary, gdalinfo --format NISAR, not only from conda list. Reinstalls fail silently.

Pin the mask state explicitly with -oo MASK=YES or -oo MASK=NO. The default is NO and it was once YES.

Copy layer paths from SUBDATASET_n_NAME output rather than from documentation, this document included; use FREQ/POL only where the product allows.

Check that the reported CRS is a real projected system and that pixel size is a plausible ground distance. A pixel size near one unit means the GeoTransform is missing.

Check corner coordinates in degrees against where you expect the granule to be.

For NEB and other calibration-grid rasters, verify the GeoTransform specifically. Different corners from the principal grid are expected; a one-unit pixel size is not.

For phase, offsets and connected components, confirm your resampling is nearest neighbour and that NaN nodata is set on both sides of the warp.

Do not quote gdalinfo -stats / -approx_stats mean and standard deviation unless the dataset carries mean_value / sample_stddev attributes; they may be synthesised from min and max.

For metadata cubes, decide whether you want a single height band or a DEM-interpolated raster. They are not interchangeable, and interpolation is GCOV and GUNW only.

For L1 frequency-B rasters, verify GCP geolocation independently.

For SME2, verify CRS and GeoTransform before overlaying; it is the least exercised product.

# 16. Quick reference

$GRANULE stands for any granule path; $GCOV, $GSLC, $GUNW and $DEM are the variables set in section 1.1.

# what is in this granule

gdalinfo NISAR:"$GRANULE"

gdalinfo NISAR:"$GRANULE" | grep HHHH

# which driver am I running

gdalinfo --format NISAR | grep DRIVER_VERSION

# full metadata (both flags required)

gdalinfo -mdd all -oo METADATA=ALL NISAR:"$GRANULE"

# open a principal layer semantically (RSLC, GSLC, GCOV, SME2)

gdalinfo -oo FREQ=A -oo POL=HHHH NISAR:"$GCOV"

# open a nested layer by full path (GUNW, GOFF)

gdalinfo NISAR:"$GUNW":/science/LSAR/GUNW/grids/frequencyA/unwrappedInterferogram/HH/unwrappedPhase

# extract to GeoTIFF / COG

gdal_translate -oo FREQ=A -oo POL=HHHH -oo MASK=YES NISAR:"$GCOV" out.tif

gdal_translate -of COG -co COMPRESS=DEFLATE NISAR:"$GCOV":/science/LSAR/GCOV/grids/frequencyA/HHHH out_cog.tif

# warp a geocoded product to lat-lon

gdalwarp -t_srs EPSG:4326 -r near -srcnodata nan -dstnodata nan \

NISAR:"$GUNW":/science/LSAR/GUNW/grids/frequencyA/unwrappedInterferogram/HH/unwrappedPhase out_ll.tif

# geocode an L1 raster from its GCPs (quick look only)

gdalwarp -t_srs EPSG:4326 -tps -r cubic NISAR:"$RSLC":/science/LSAR/RSLC/swaths/frequencyA/HH rslc_ll.tif

# amplitude / dB of a layer, no code required

gdal_translate "DERIVED_SUBDATASET:AMPLITUDE:NISAR:$GSLC:/science/LSAR/GSLC/grids/frequencyA/HH" amp.tif

gdal_translate "DERIVED_SUBDATASET:LOGAMPLITUDE:NISAR:$GCOV:/science/LSAR/GCOV/grids/frequencyA/HHHH" db.tif

# interpolate a metadata cube onto the imaging grid

gdal_translate -oo QUANTITY=incidenceAngle -oo DEM_FILE="$DEM" \

NISAR:"$GCOV":/science/LSAR/GCOV/metadata/radarGrid/incidenceAngle inc.tif

# single pixel

gdallocationinfo NISAR:"$GRANULE":<path> <pixel> <line>

gdallocationinfo -wgs84 NISAR:"$GRANULE":<path> <lon> <lat>

# batch tuning from S3

export NISAR_PREFETCH_GRID=24 GDAL_CACHEMAX=2048 GDAL_NUM_THREADS=ALL_CPUS

# what is the driver doing

CPL_DEBUG=NISAR_DRIVER gdalinfo -oo FREQ=A -oo POL=HHHH NISAR:"$GRANULE"